1.研究背景与技术演进概述

声源定位（Sound Source Localization,SSL）是语音交互、机器人自主导航、会议系统、智能监控与增强现实等领域的核心支撑技术。随着深度学习的发展，SSL研究逐渐从传统依赖麦克风几何先验的特征构建方法，演进至可端到端学习声学空间映射关系的深度神经网络模型。麦克风阵列的形态也从规则阵列拓展到分布式阵列、随机非均匀阵列，推动了模型结构与特征形式的多样化[1]。

1.1硬件基础与数据采集接口

在底层硬件方面，阵列数据的采集效率和质量至关重要。文献[2]详细对比了PDM（脉冲密度调制）与I2S两种主流数字麦克风接口的差异。PDM因其单比特流、高时钟频率和强抗电磁干扰能力，尤其适用于构建低功耗、高密度的MEMS数字麦克风阵列。然而，PDM数据必须经过数字滤波器（如CIC滤波器[3]和半带滤波器）解调转换为PCM数据才能进行后续的信号处理，这一过程在FPGA上的高效实现是构建实时阵列的基础。相比之下，I2S接口传输PCM多比特流，常用于高保真音频传输，但功耗和布线复杂度相对较高。本课题的研究方向倾向于低功耗嵌入式部署，因此对PDM/I2S的接口特性及其FPGA实现进行了深入调研。

1.2传统信号处理方法的演进

在深度学习尚未普及之前，基于时延估计（TDOA）和波束形成（Beamforming）的信号处理方法占据主流。在传统方法方面，基于广义互相关相位变换（GCC-PHAT）与声源能量累积的SRP-PHAT方法[4]长期是SSL的主流技术。该方法通过搜索空间网格上的最大功率响应来确定声源，在低混响环境下表现优异。然而，其性能高度依赖阵列朝向、麦克风间距以及噪声与混响条件，尤其在实际部署中容易出现主峰模糊、伪峰增强等问题[5]。

为此，大量改进型SRP算法相继提出。在分布式声学网络（WASN）中，传统集中式处理面临巨大的通信开销和同步难题。D. Salvati等[6]人提出的V-SRP最大池化（Max-Pooling）操作，旨在抑制非目标位置的噪声和混响成分，使系统在混响时间RT60=0.8s的情况下，准确率（AR）相比传统方法提升了约15%,Zhu等[7]人提出的SRP-LMS（基于最小均方误差的波束形成功率图）作为一种与经典SRP-PHAT互补的新特征，通过最小化波束成形误差在强混响环境下展现出更强的鲁棒性，但低信噪比（SNR）时易受干扰。如何对这些互补特征进行动态加权融合，成为本课题特征构建的关键出发点。

2.深度学习驱动的声源定位技术发展

深度学习的引入使SSL能够在无需显式几何约束的情况下，通过大量数据自主学习麦克风阵列结构与声学空间映射关系，极大地提升了模型在复杂声学环境中的性能。

2.1端到端特征学习

早期的深度学习方法主要以手工设计的特征（如梅尔频谱、GCC-PHAT）作为输入。为了实现更有效的端到端学习，研究人员开始关注直接处理原始波形的方法。M. Ravanelli等[8]提出的SincNet架构通过将传统CNN的第一层卷积核替换为可学习的参数化Sinc函数，使其具备明确的带通滤波特性，有效捕捉了语音信号的物理特征。J. Jung等[9]人提出的RawNet进一步发展了这一思路，利用GRU和残差结构，在原始波形上实现了高性能的说话人识别与定位。Axel Berg, Jens Gulin等[10]提出的wav2pos模型则将SSL任务重构为集合到集合（Set-to-Set）的回归问题，并利用Masked Autoencoders(MAE)和Transformer结构，实现了对3D定位的高精度预测，并对麦克风位置缺失具有鲁棒性。这种对原始信号或波形进行深度分析的思路，证明了数据驱动特征学习的优越性。

2.2分布式网络与图神经网络

在非结构化的分布式阵列中，如何有效协作是关键。Tango算法通过节点间压缩信号的交换实现高效的分布式去噪[11]。为了应对阵列拓扑动态变化的挑战，图神经网络（GNN）被应用于SSL任务[12]。GNN-SSL框架将麦克风视为图节点，通过学习节点间的成对关系（Pairwise Relations）来聚合全局特征，实现了与阵列几何结构无关的鲁棒定位[13]。在时钟同步方面，基于扩散的分布式多帧卡尔曼滤波（DD-MFKF）算法通过邻居信息扩散与自适应权重，解决了大规模WASN中的时钟漂移和定位跟踪问题[14]。此外，还有研究通过优化传感器选择策略，在保证定位精度的前提下显著降低了通信开销[15]。

2.3 3D定位网络与声源距离估计（SDE）和事件分类（SED）

事件分类

在处理多声源场景和声音事件定位（SELD）问题上，卷积循环神经网络（CRNN）[16]展现出强大的时序建模能力。SELDNet[17]通过并行分支同时输出声音事件检测（SED）和方向估计（DOA），成为该领域的基准模型。针对复杂的多重声源重叠和环境感知问题，Adrenaline网络[18]采用了基于注意力的Seq2Seq结构，通过编码器-解码器捕捉长时序依赖，显著提高了定位的稳定性。

混响特征与距离感知

在更精确的3D定位方面（同时估计角度和距离），传统方法对混响特征的利用不足。最新的研究开始关注直达声与混响声比率（DRR）和短时自相关功率（stpACC）等[19]特征，以显式地捕捉距离线索。此外，将几何先验融入网络设计成为新的趋势[20]。

单通道距离估计与特征建模

E. Georganti等人[21]首次提出单麦克风在室内环境中检测说话人绝对距离（0m-3m）的方法，解决传统方法需麦克风阵列或双耳信号的限制，在无法部署阵列的场景下，基于CRNN的单通道距离估计[22]展现了潜力。为避免相位缠绕问题，系统采用sin/cos(STFT相位)自适应空间注意力模块，在1-14米的连续距离估计中实现了22 cm的平均绝对误差（MAE）。对于熟悉声源如人声，LP残差峰值比、频谱偏度等8种距离敏感特征被用于构建GMM分类器，在实际房间中展现出优于人类听觉感知的判断力。

2.4传统CNN的DOA估计与球面卷积

为解决高混响环境下的单声源方向估计（DOA）与跟踪问题以及传统方法（如SRP-PHAT）在噪声和混响场景中性能急剧下降，且无法实时跟踪动态声源，David Diaz-Guerra等人[23]提出Cross3D模型依赖于紧凑阵列的麦克风，相较于SELDnet以及wav2pos来说其算法模型参数量少，仅仅做DOA估计更适合实际工程。

针对空间定位数据本质上位于球面上的特点，传统二维卷积（如将声源图展平）会带来极点处的几何畸变和不规则采样问题[24]。David Diaz-Guerra等人[25]继续在Cross3D的基础上提出IcoCNN（二十面体卷积）用于解决这一问题。Zhu等人提出的IFAN（Icosahedral Feature Attention Network）[7]系统性地结合了IcoCNN、双分支特征（SRP-PHAT与SRP-LMS）以及特征注意力框架[26]，证明了基于二十面体划分的球面卷积相比传统3D CNN在参数量和FLOPs上均有优势（仅0.07M参数，6.36M FLOPs），并在强混响环境中表现出更优异的性能。IFAN的成功验证了后续本课题“SRP特征融合”与“IcoCNN”结合的可行性和创新性。

3 硬件部署与模型优化

高性能的深度SSL模型面临巨大的计算量和延迟，难以在边缘设备上部署。因此，FPGA因其高并行性和低延迟特性成为研究热点。Neural ODE与资源复用：受神经常微分方程（Neural ODE）模型的启发[29]，通过时分复用单一计算单元来模拟深层网络的行为，以极小的资源代价换取网络深度的增加，但这种成本效益极高的思路，可行性仍需要进一步验证。

SAF-CNN框架[27]提出了一种结构化剪枝与细粒度块划分策略，有效地将稀疏的CNN模型映射到FPGA上，解决了稀疏矩阵运算的负载不均衡问题。hls4ml等工具链[28]的发展使得将高层框架（如Keras/PyTorch）的模型转换为HLS代码成为可能，进一步加速了硬件原型设计。

Jun Yin, Marian[30]在25年提出LC-SRP-Edge 与 Cross3D-Edge以解决如何在资源极度受限的边缘设备上，实现高鲁棒性的声源定位。尽管基于SRP-PHAT特征和深度神经网络的方法在嘈杂和混响环境中表现出色，但其巨大的计算量和模型尺寸使其难以在实时边缘应用中部署。

参考文献：

- 张结,呼德,张晓雷,等.分布式麦克风阵列拾音理论与应用方法综述[J].数据采集与处理,2024,39(05):1085-1113.

- 刘晓林.基于FPGA的麦克风阵列声音采集系统及其声成像方法研究[D].东北大学,2022.

- S. Gupta, "CIC filter implementation on FPGA for audio applications," IEEE Transactions on Circuits and Systems, 2020.

- D. Diaz-Guerra et al., "Robust Sound Source Tracking Using SRP-PHAT and 3D Convolutional Neural Networks," IEEE/ACM Trans. Audio, Speech, Lang. Process., 2020.

- A. Bertrand and M. Moonen, "Distributed adaptive node-specific signal estimation in fully connected sensor networks—Part I: Sequential node updating," IEEE Trans. Signal Process., vol. 58, no. 10, pp. 5277–5291, 2010.

- D. Salvati, C. Drioli and G. L. Foresti, "Acoustic Source Localization Using a Geometrically Sampled Grid SRP-PHAT Algorithm With Max-Pooling Operation," in IEEE Signal Processing Letters, vol. 29, pp. 1828-1832, 2022

- X.-C. Zhu et al., "IFAN: An Icosahedral Feature Attention Network for Sound Source Localization," IEEE Trans. Instrum. Meas., 2024.

- M. Ravanelli and Y. Bengio, "Speaker Recognition from Raw Waveform with SincNet," Proc. IEEE Spoken Lang. Technol. Workshop (SLT), 2018.

- J. Jung et al., "RawNet: Advanced end-to-end deep neural network using raw waveforms for text-independent speaker verification," Proc. Interspeech, 2019.

- Axel. Berg, J. Gulin, et al., "wav2pos: Sound Source Localization using Masked Autoencoders," Proc. ICASSP, 2024.

- N. Furnon, R. Serizel, S. Essid and I. Illina, "DNN-Based Mask Estimation for Distributed Speech Enhancement in Spatially Unconstrained Microphone Arrays," in IEEE/ACM Transactions on Audio, Speech, and Language Processing, vol. 29, pp. 2310-2323, 2021

- E. Grinstein, M. Brookes and P. A. Naylor, "Graph Neural Networks for Sound Source Localization on Distributed Microphone Networks," ICASSP 2023 - 2023 IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), Rhodes Island, Greece, 2023, pp. 1-5

- Q. Zhang, Y. Zhang, and Y. Lo, "Diffusion-Based Distributed Multi-Frame Kalman Filtering With Speech Distortionless Constraint for Speech Enhancement," IEEE/ACM Trans. Audio, Speech, Lang. Process., 2022.

- D. Hu et al., "Distributed energy-saving speech enhancement in wireless acoustic sensor networks," Information Fusion, 2024.

- J. Wei et al., "Real-Time Sound Event Localization and Detection: Deployment Challenges on Edge Devices," arXiv preprint arXiv:2409.11700, 2024.

- M. Neri, A. Politis, D. Krause, M. Carli and T. Virtanen, "Single-Channel Speaker Distance Estimation in Reverberant Environments," 2023 IEEE Workshop on Applications of Signal Processing to Audio and Acoustics (WASPAA), New Paltz, NY, USA, 2023, pp. 1-5

- S. Adavanne, A. Politis, J. Nikunen, and T. Virtanen, "Sound Event Localization and Detection of Overlapping Sources Using Convolutional Recurrent Neural Networks," IEEE J. Sel. Topics Signal Process., vol. 13, no. 1, pp. 34–48, 2019.

- C. Schymura et al., "Exploiting Attention-based Sequence-to-Sequence Architectures for Sound Event Localization," 2020 28th European Signal Processing Conference (EUSIPCO), Amsterdam, Netherlands, 2021, pp. 231-235

- D. Berghi, "Reverberation-based Features for Sound Event Localization and Detection with Distance Estimation," Proc. ICASSP, 2025.

- T. Gburrek, J. Schmalenstroeer, A. Brendel, W. Kellermann and R. Haeb-Umbach, "Deep Neural Network based Distance Estimation for Geometry Calibration in Acoustic Sensor Networks," 2020 28th European Signal Processing Conference (EUSIPCO), Amsterdam, Netherlands, 2021, pp. 196-200,

- E. Georganti, T. May, S. van de Par, A. Harma and J. Mourjopoulos, "Speaker Distance Detection Using a Single Microphone," in IEEE Transactions on Audio, Speech, and Language Processing, vol. 19, no. 7, pp. 1949-1961, Sept. 2011

- M. Neri, A. Politis, D. A. Krause, M. Carli and T. Virtanen, "Speaker Distance Estimation in Enclosures From Single-Channel Audio," in IEEE/ACM Transactions on Audio, Speech, and Language Processing, vol. 32, pp. 2242-2254, 2024

- D. Diaz-Guerra, A. Miguel and J. R. Beltran, "Robust Sound Source Tracking Using SRP-PHAT and 3D Convolutional Neural Networks," in IEEE/ACM Transactions on Audio, Speech, and Language Processing, vol. 29, pp. 300-311, 2021

- S. Chakrabarty and E. A. P. Habets, "Multi-Speaker DOA Estimation Using Deep Convolutional Networks Trained With Noise Signals," IEEE J. Sel. Topics Signal Process., 2019.

- D. Diaz-Guerra, A. Miguel and J. R. Beltran, "Direction of Arrival Estimation of Sound Sources Using Icosahedral CNNs," in IEEE/ACM Transactions on Audio, Speech, and Language Processing, vol. 31, pp. 313-321, 2023

- W. Mack et al., "Signal-Aware Broadband DOA Estimation Using Attention Mechanisms," Proc. ICASSP, 2020.

- Nankai Univ. & CAS, "SAF-CNN: A Sparse Acceleration Framework of Convolutional Neural Network for Embedded FPGAs," Journal of Computer Research and Development, 2023.

- B. Ramhorst, V. Lončar and G. A. Constantinides, "FPGA Resource-aware Structured Pruning for Real-Time Neural Networks," 2023 International Conference on Field Programmable Technology (ICFPT), Yokohama, Japan, 2023, pp. 282-283

- A. Cost-Efficient, "FPGA-Based CNN-Transformer using Neural ODE," Proc. DATE, 2024.

- Jun Yin, Marian “Verhelst,CNN-based Robust Sound Source Localization with SRP-PHAT for the Extreme Edge,”ACM Transactions on Embedded Computing Systems, Volume 22, Issue 3,03,2025